Aurora MHD Technology Set

Architecture 1 — Corkscrew MHD Accelerator

with Induction Recovery (Aerospace IADS)

Prior Art Synthesis & Strategic Position

Working document — internal use

# 1. Executive Summary

Architecture 1 is a high-power J×B Lorentz force MHD plasma accelerator with downstream induction-recovery generator section, configured in a helical corkscrew duct geometry within a ~5.5 m enclosure. The plasma working fluid is HydroSynth-ionized atmospheric or seeded gas, accelerated to 5-7 km/s under combined magnetic field of 10 T toroidal and 3 T poloidal (modern HTS). The architecture targets the aerospace IADS market as primary, with parallel applications in commercial hypersonic flow-management and (longer-term) space-power propulsion.

Architecture 1 sits at the intersection of three heritage threads: AJAX-class hypersonic MHD bypass (Russian Leninets, NASA Ames/Glenn, MARIAH II), MPDT applied-field accelerators (Princeton EPPDyL, Stuttgart, Chinese groups), and marine MHD propulsion (Yamato-1, the only full-scale operational MHD-propelled vehicle). It is a high-risk, high-upside bet that depends on Aurora succeeding at three things prior programs did not: closing the AJAX power balance, escaping the Kerrebrock constraint via HydroSynth, and demonstrating the corkscrew helical geometry delivers what coaxial topologies could not.

The four-phase prior art exercise establishes that Architecture 1 has eight specific risks, five of which involve fundamental physics questions that 30-60 years of focused research have not definitively resolved. However, modern adjacent advances (HTS magnets, aerospace AM, SiC/GaN pulsed power, commercial hypersonic ecosystem, IADS market timing per EO 14186) have transformed the commercial viability landscape since MARIAH II was terminated in 2007. The recommended primary end-state is IADS directed-energy capability, with commercial flow-management as parallel secondary track.

# 2. Defensible Technical Claims

The claims below can be defended with reference to documented prior art. Each is followed by its supporting evidence and any caveats.

| Claim | Evidence and Caveat |
| --- | --- |
| J×B Lorentz force acceleration is mature physics | MPDT heritage 60+ years; Japanese SFU 1996 quasi-steady pulsed flight; Chinese 150 kW SC-MPDT at 76.6% efficiency; Zheng et al. 12 kW HTS-AFMPDT first-of-kind 2025. Caveat: no multi-MW continuous operational MPDT in 60 years. |
| AF-MPDT applied-field topology aligns with 10T+3T configuration | AF-MPDT axial+radial field is topological analog. Caveat: flying AF-MPDTs operate <1 T central plasma; 10-15 T is order-of-magnitude extrapolation. |
| Full-scale MHD propulsion has been demonstrated operationally | Yamato-1 (1992), 30 m ship, 4 T NbTi magnets, carried humans, 8 kN per thruster. Caveat: 15% efficiency drove commercial obsolescence. |
| TsAGI confirmed accelerator + downstream generator topology | TsAGI 'high-enthalpy MHD generator using accelerator exhaust as working medium' is Architecture 1's exact topology. Caveat: heritage in Russian-language primary literature, partially translated. |
| Modern HTS magnets retire the magnet-mass-and-cost constraint | Conduction-cooled HTS at 40 K: power 285 kW → <1 kW; mass 220 kg → 60 kg (Zheng 2025). CFS 20 T at 20 K validated. Caveat: integration into helical duct still requires custom engineering. |
| Modern aerospace AM makes corkscrew helical duct fabricable | NASA RAMPT 102 cm × 97 cm with integrated channels in 30 days. GRCop-42/84 chambers: 30,000+ s, 400+ starts hot-fire. Cellcore helical/lattice cooling. Caveat: specific Architecture 1 design must be validated in production. |
| SiC/GaN pulsed power makes HydroSynth-class drives commercial COTS | Nanosecond-class HV pulse generation now solid-state commercial. Caveat: HydroSynth's specific σ × volume × duration requirement at MHD-relevant scale remains unproven. |
| NS-DBD plasma physics is flight-qualified for surface effects | Full-size piloted glider, UAVs, hypersonic Mach 6 quiet tunnel demos. Caveat: bulk-volumetric ionization at MHD-relevant σ has not been demonstrated in any flying or operational system in 20+ years. |
| Commercial hypersonic ecosystem provides customer base | Hermeus ($1B valuation), Stratolaunch, Venus Aerospace, Hypersonix, Boom Supersonic. $9B+ defense tech VC 2024. EO 14186, scaled hypersonics designation. Caveat: market timing is favorable today but may not be in 5-10 years. |
| Acquisition exit pathway established at validated benchmarks | AeroVironment $4.1B BlueHalo (late 2024) for laser/electromagnetic systems. Implies $1-5B for Architecture 1 at proven-demonstrator stage. Caveat: capture requires reaching demonstrator stage, not just category. |

# 3. Acknowledged Risks

The risks below are not retired by prior art. Five involve fundamental physics questions that 30-60 years of focused research have not definitively answered. Architecture 1 must address each through its development program.

| Risk | Evidence and Retirement Path |
| --- | --- |
| Risk 1 — The AJAX Power Balance Problem (HIGHEST PRIORITY) | 30-year unresolved disagreement between NASA Ames (parasitic ionization power exceeds generated) and Russian Sheikin/Kuranov (self-sustained operation possible). No hardware has settled this. Retired by: rigorous power balance analysis specific to Architecture 1 configuration with HydroSynth, recovery section, and corkscrew geometry. STAGE 1 PRE-HARDWARE DELIVERABLE. |
| Risk 2 — The Kerrebrock Constraint | 60-year fundamental physics constraint: non-equilibrium ionization requires Hall-effect conditions creating dominant electrical losses. HydroSynth's bulk-volumetric DBD attempts to escape this. Retired by: HydroSynth σ characterization document benchmarking against DBD literature, NETL seedless DPE, Tokyo Tech RF. STAGE 1 PRE-HARDWARE DELIVERABLE. |
| Risk 3 — Cathode/Electrode Erosion at High Current Density | 60-year MPDT problem since 1986. >100 A/cm² causes evaporation, sublimation, melting, cracking across all geometries. Partial solutions exist (Li/Ba propellant, hollow cathodes, ThO₂ doping) but no continuous multi-MW operational MPDT exists. Retired by: multi-thousand-hour J×B-mode electrode test program ($5-15M, 2-3 years; parallel with Architecture 4 testing per CC-37). |
| Risk 4 — DBD Bulk-Volumetric Ionization at MHD-Relevant σ | 20+ years of research; flight-qualified for surface effects only. HydroSynth, NETL seedless DPE, Tokyo Tech RF all pursue same gap; none has demonstrated MWe-class. Retired by: sub-scale (kW class) HydroSynth + MHD channel testing for σ verification at Architecture 1 spatial scales. |
| Risk 5 — Full-System Mass-Power Ratio for IADS Deployment | AJAX IVTAN studies showed MHD generator + magnet mass is significant fraction of vehicle mass. Modern HTS helps but full-system mass budget unexamined for Architecture 1. Retired by: full-system mass-power model for representative IADS scale (5-50 MW). STAGE 1 PRE-HARDWARE DELIVERABLE. |
| Risk 6 — Mach-Independence Ceiling (Macheret/Shneider) | Inter-segment electrode arcing flattens expected interaction parameter scaling with Mach number. Retired by: Architecture 1 electrode segmentation design study using modern AM (CC-24, CC-41) addressing arcing constraint. |
| Risk 7 — Helical Corkscrew Geometry Performance Justification | No prior MHD accelerator uses helical corkscrew duct. AF-MPDT swirl mechanism is closest analog but geometrically different. Retired by: corkscrew design rationale document with CFD-MHD modeling (Marzouk OpenFOAM solver) showing performance vs coaxial baseline. |
| Risk 8 — End-State Strategic Choice | Three distinct end-states (IADS, flow-management, space-power variant) imply different development priorities. Spreading resources across three paths is organizational risk. Retired by: explicit primary end-state choice with documented rationale (recommended Path A: IADS, in Section 5). |

# 4. Risk Retirement Development Plan

Sequenced development program addressing the eight risks. The Stage 1 analytical work delivers three pre-hardware deliverables that resolve fundamental physics questions before any hardware commitment.

### Stage 0 — Strategic Foundation (months 0–3, ~$0.5M)

- End-state choice with documented rationale (Risk 8) — recommended Path A: IADS

- Engagement strategy with primary end-state customer category

- IP landscape review for corkscrew helical duct geometry

- Technology scouting relationships: Hermeus, Stratolaunch, Venus (commercial hypersonic); AeroVironment, L3Harris, RTX, Lockheed, Northrop (defense primes); Ad Astra, Magdrive, Phase Four (plasma propulsion partners)

### Stage 1 — Analytical Risk Retirement (months 3–9, ~$1.5–2.5M)

The three pre-hardware deliverables:

- Rigorous power balance analysis anchored to Bruno-Czysz framework (CC-31), with explicit treatment of HydroSynth parasitic load, recovery section, and corkscrew geometry coupling. Resolves AJAX 30-year debate for Architecture 1 specific configuration. (Risk 1)

- HydroSynth σ characterization document benchmarking against DBD literature, NETL seedless DPE, Tokyo Tech RF at Architecture 1's required σ × volume × duration combination. (Risk 2, 4)

- Full-system mass-power model for representative IADS deployment scale (5-50 MW), translating component-level advances to operational vehicle ratio. (Risk 5)

Plus parallel work:

- Architecture 1 σ × B² × u design space mapping using Marzouk OpenFOAM solver

- Corkscrew geometry CFD-MHD modeling vs coaxial baseline (Risk 7)

- Electrode segmentation design study addressing Mach-independence ceiling (Risk 6)

- HTS magnet engagement with CFS / Tokamak Energy / Stuttgart IRS

- AM electrode design with NASA RAMPT/RAMFIRE-class vendors

### Stage 2 — Component-Level Validation (months 9–30, ~$8–18M)

- Sub-scale (kW class) HydroSynth + MHD channel testing for σ verification (Risk 4 entry; combines with Risk 3)

- Multi-thousand-hour J×B mode electrode lifetime test program with AM-fabricated refractory candidates (Risk 3)

- Pre-ionization power supply integration testing

- AF-MPDT engagement with Stuttgart IRS and Princeton EPPDyL for shared test infrastructure

- Sub-scale corkscrew duct fabrication via NASA RAMPT/RAMFIRE-class AM

### Stage 3 — Sub-Scale Integrated Demonstrator (months 30–60, ~$50–120M)

- 1–5 MW class Architecture 1 demonstrator: J×B accelerator + helical duct + induction recovery + HydroSynth + HTS magnet

- Validates: continuous operation, supersonic helical flow, HydroSynth at bulk-volume σ, electrode lifetime, magnet integration, power balance at scale

- This is Architecture 1's MARIAH II equivalent at 40-100× smaller scale — what makes FOAK risk tractable

### Stage 4 — Operational Pilot (months 60–120, $300M–1B)

- Path A (IADS, recommended primary): operational test article for defense customer; establishes TRL 6-7; positions for prime acquisition

- Path B (flow-management): integration with Hermeus/Stratolaunch/Venus vehicle as flow-management component

- Path C (space-power variant): engagement with Ad Astra/SpaceNukes for nuclear-electric variant

### Stage 5 — Strategic Exit / Scale (year 10+)

- Acquisition by defense prime at $1-5B order of magnitude (BlueHalo benchmark)

- Or independent scaling for Path B/C variants

# 5. End-State Strategic Choice

Phase 4 surfaced three distinct candidate end-states. The synthesis must choose one as primary; spreading scarce engineering resources across three product paths would prevent any one from succeeding.

| Dimension | Path A: IADS | Path B: Flow-Mgmt | Path C: Space-Power | Notes |
| --- | --- | --- | --- | --- |
| Customer | DOD + allies | Hermeus, Stratolaunch | Ad Astra, SpaceNukes | Path A largest market |
| Market timing | Exceptional (EO 14186) | Favorable | Niche | Path A best timing |
| Funding pathway | DOD + defense VC | Commercial VC + customer | Strategic partnership | Path A most channels |
| Exit value | $1-5B (BlueHalo) | $200M-1B | $50-300M | Path A 5-30× others |
| Tech fit | Strong | Moderate | Moderate | Path A best alignment |
| Reg. complexity | High (ITAR) | Medium | Medium | Path A higher cost |
| Time to revenue | 7-10 yrs | 4-6 yrs | 8-12 yrs | Path B fastest |
| Engineering risk | Highest | Lower | Higher | Path B safest |
| TAM | $50B+ US IADS | $10-20B hypersonic | $5-10B space prop. | Path A largest |

Recommended Primary End-State: Path A — IADS Directed-Energy Capability

The recommendation is based on five factors:

- Highest exit value benchmark — BlueHalo $4.1B precedent implies $1-5B for Architecture 1 at proven-demonstrator stage; 5-10× Path B, 10-30× Path C

- Largest TAM — US IADS market plus allied-nation parallel investment ($50B+ over 10 years)

- Best market timing — EO 14186, scaled hypersonics designation, $9B+ defense tech VC 2024

- Best alignment with Architecture 1 capabilities — high-thrust J×B accelerator with integrated power management is what IADS directed-energy applications value

- Strongest funding path — AFRL, AFOSR, DARPA, DIU, OUSD(R&E) all have active programs Architecture 1 fits directly

Path B (flow-management) should be developed as parallel secondary track — it validates Architecture 1's flow-control capability with shorter feedback loop, Hermeus HEAT facility provides accessible test infrastructure, successful demonstrations strengthen the IADS pitch, commercial revenue can co-fund IADS development, and the technical work is ~80% shared between Path A and Path B.

Path C (space-power variant) should be deferred until Stage 3 demonstrator success on Path A. Ad Astra-SpaceNukes alliance is the natural partner; engagement should follow Architecture 1 proving multi-MW continuous operation.

# 6. What Architecture 1 Is Not

- Not a substitute for conventional jet engines in commodity flight applications

- Not a near-term commercial product — Stage 1 to Stage 4 runs ~10 years

- Not de-risked — eight specific risks remain, including fundamental physics questions unresolved for 30-60 years

- Not a single-architecture bet — benefits significantly from cross-cutting work shared with other Aurora architectures (HTS, AM, pulsed power, electrode lifetime)

- Not the only J×B accelerator concept being pursued in 2025 — modern HTS-AFMPDT (Princeton, Stuttgart, Chinese), commercial plasma propulsion (Ad Astra, Magdrive, Phase Four), AJAX-class hypersonic propulsion all active. Architecture 1 must articulate specific differentiation.

# 7. Architecture 1 Prior Art Snapshot

Single-page reference summary of where each design dimension stands after the four-phase prior art exercise.

| Design Dimension | Status | Source / Action |
| --- | --- | --- |
| J×B Lorentz force acceleration | Demonstrated (sub-MW) | MPDT, AFMPDT 60-year |
| AF applied-field topology | Operationally proven | HTS-AFMPDT 2025 |
| Full-scale MHD propulsion | Demonstrated (Yamato-1) | 1992, 15% efficiency |
| Accelerator + recovery topology | Demonstrated (TsAGI) | Russian heritage |
| Modern HTS magnets at 10-15 T | Commercially validated | CFS, Tokamak Energy, Zheng |
| Aerospace AM for helical ducts | Production-ready | NASA RAMPT/RAMFIRE |
| Pre-ionization power supply | Commercial COTS | SiC/GaN industrial plasma |
| NS-DBD plasma physics (surface) | Flight-qualified | Multiple programs |
| Corkscrew helical geometry | Novel | No direct prior art |
| HydroSynth bulk-volumetric σ | Unproven at scale | Risk 4 — 20-year gap |
| Power balance closure | Unresolved (30 yr) | Risk 1 — Stage 1 deliverable |
| Kerrebrock constraint escape | Unproven | Risk 2 — Stage 1 deliverable |
| Cathode lifetime at high J/A² | 60-year unsolved | Risk 3 — multi-1000 hr testing |
| Mass-power ratio (IADS scale) | Unmodeled | Risk 5 — Stage 1 deliverable |
| Mach-independence ceiling | Bounded but real | Risk 6 — design study |
| Customer ecosystem | Newly created | Hermeus + DOD |
| Acquisition exit pathway | Validated benchmark | BlueHalo $4.1B |
| End-state choice | Ambiguous | Risk 8 — Stage 0 (Path A recommended) |

Net Assessment

Architecture 1 is a high-risk, high-upside engineering bet. The 30-60 year unresolved physics questions (power balance, Kerrebrock, electrode erosion at high J/A², bulk-volume σ via DBD) must be addressed analytically in Stage 1 before hardware commitment. Modern adjacent advances (HTS, AM, pulsed power, commercial hypersonic ecosystem, IADS market timing) make the commercial path tractable in a way it was not when MARIAH II was terminated in 2007.

The recommended primary end-state (Path A: IADS directed-energy capability) takes advantage of exceptional market timing, largest TAM, and best alignment with Architecture 1's specific capabilities. Path B (flow-management) should be developed as parallel secondary track. Path C (space-power variant) is a future opportunity post-Stage 3.

The strategic case rests on (a) successful Stage 1 analytical risk retirement of the three pre-hardware deliverables, (b) modern technology adjacencies delivering on their promise during Stage 2 component validation, and (c) DOD funding alignment with EO 14186 and the scaled-hypersonics designation through Stage 3.

This synthesis derives from a four-phase prior art landscape covering: 9 direct heritage entries, 8 supporting work entries, 10 disrupting work entries, 6 adjacent inspirational entries, plus 43 cross-cutting register items spanning the broader Aurora technology set. The full prior art document accompanies this synthesis as a working reference.