Aurora MHD Technology Set

Architecture 4 — Supersonic Closed-Cycle Faraday MHD

Prior Art Synthesis & Strategic Position

Working document — internal use

# 1. Executive Summary

Architecture 4 is a closed-cycle Brayton MHD generator targeting the 5–50 MW stationary mid-scale power market. Its operating point: working gas (N₂ or noble gas, currently under review) seeded with cesium, accelerated to supersonic Mach 1.5–3 through a converging-diverging nozzle, expanded through a segmented-Faraday MHD channel at B = 10–15 T (modern REBCO HTS), with heat input via ceramic regenerator. Target: 50–55% plant efficiency, $2,000–3,500/kWe capex landing zone.

The architecture is a recombination of established sub-system capabilities in a configuration that has not been built before. It makes two specific extensions beyond demonstrated prior art: supersonic operation in closed cycle (NASA Lewis stopped at Mach 0.55; Sakhalin demonstrated supersonic but pulsed) and continuous multi-MW non-equilibrium σ enhancement (Tokyo Tech demonstrated the physics sub-MW). These extensions are bounded and identifiable — not the broad ‘unresolved technical issues’ that ended the 1993 DOE program.

The four-phase prior art exercise establishes that Architecture 4 is a defensible engineering bet with seven specific, addressable risks. The strategic case rests on customer-facing market validation in early development stages and on cost discipline to keep first-of-a-kind capex within the $2,000–3,500/kWe landing zone. Architecture 4 is positioned as a competitive, not dominant, mid-scale stationary power option.

# 2. Defensible Technical Claims

The claims below can be defended with reference to documented prior art. Each claim is followed by its supporting evidence and any caveats.

| Claim | Evidence and Caveat |
| --- | --- |
| Closed-cycle Faraday MHD physics works | NASA Lewis demonstrated 92% of ideal Faraday voltage (141 V at 1.7 T, He-Cs working fluid). Caveat: sub-MW class only, M = 0.55 max. |
| Multi-hour MHD operation is achievable | U-25 channel #2 disassembled after 100+ hours; CDIF set record durations; ITC POC channel design target was 2,000 hours. Caveat: all open-cycle; closed-cycle Cs-vapor duration is a gap. |
| σ enhancement via non-equilibrium ionization is physically achievable | Tokyo Tech RF-assisted CCMHD; Litchford/Harada NASA Marshall analysis; Marzouk 2024 σ model. Caveat: sub-MW only; NASA Lewis explicitly noted no nonequilibrium power generated at the multi-electrode scale. |
| Closed-cycle eliminates the dominant historical failure mode | Slag chemistry, anode pitting, and seed depletion accounted for the largest portion of channel failures in coal-MHD heritage. Architecture 4 has no slag. Caveat: introduces different failure mode (Cs-vapor corrosion) for which no >1,000 hour data exists. |
| Required magnet field strength is commercially supported | CFS validated 20 T at 20 K REBCO HTS with DOE acceptance; Tokamak Energy validated 26.2 T at 4 K and complete magnet system. Caveat: integration into flowing-plasma channel requires custom thermal management engineering. |
| Modern multiphysics CFD design capability is available | Marzouk OpenFOAM MHD solver with Sakhalin validation; commercial COMSOL and ANSYS Fluent MHD modules; NETL multiphysics tool stack. Caveat: CFD does not substitute for hardware testing at scale. |
| Cs handling is established engineering practice | Accelerator photocathode community has decades of routine Cs handling at high purity; mature commercial tooling (SAES Getters AMDs, UHV protocols). Caveat: photocathode regime is room-T UHV vs Architecture 4's 1500-1900 K flowing plasma. |
| Ceramic regenerator innovation delivers material capex savings | Earlier Aurora analysis demonstrated approximately $74M savings vs fluid heat exchanger; Zauderer pebble-bed concept provides theoretical heritage. Caveat: not yet validated at integration level. |
| 50–55% plant efficiency is consistent with independent analyses | University of Florida, Nagaoka, Indian groups, Chinese groups, Tokyo Tech all converge on 50–60% CCMHD-Brayton efficiency; Litchford/Harada specifically 55.2%. Caveat: all theoretical; no multi-MW operational validation. |
| Supersonic Faraday geometry works at scale | Sakhalin PMHDG demonstrated 510 MW from a 4.5 m supersonic divergent Faraday channel. Caveat: pulsed only (2.5–10 second duration); continuous operation extension is first-of-its-kind. |

# 3. Acknowledged Risks

The risks below are the ones prior art does not retire. Architecture 4 must address them through its development program.

| Risk | Evidence and Retirement Path |
| --- | --- |
| Risk 1 — Cs-vapor electrode lifetime at multi-thousand hours undemonstrated | No closed-cycle MHD has run >1,000 hours with operational Cs exposure data. Photocathode community Cs experience does not transfer (room-T UHV vs flowing plasma at 1500–1900 K). The 240,000-hour utility lifetime expectation is two orders of magnitude beyond any demonstrated value. Highest-priority risk. Retired by: multi-thousand-hour test program on candidate electrode materials in Cs-vapor at temperature. ~$5–15M, 2–3 years. |
| Risk 2 — Supersonic closed-cycle integration never done | Sakhalin is supersonic but pulsed and open-cycle. NASA Lewis is closed-cycle but subsonic. No prior art covers Architecture 4's specific combination. Retired by: sub-scale closed-cycle supersonic demonstrator (1–5 MW class) with full instrumentation. ~$30–80M, 3–5 years. |
| Risk 3 — Multi-MW non-equilibrium σ unproven at scale | Tokyo Tech RF-assisted work is sub-MW. NASA Lewis explicitly noted no non-equilibrium power generated at scale despite analytical framework being clear. Architecture 4's 50–100 S/m via non-equilibrium is the upper edge of literature-consistent range. Retired by: σ characterization at 1 MWe-class with full electron-density and electron-temperature diagnostics. Combine with Risk 2 demonstrator. |
| Risk 4 — Working fluid choice (N₂ vs noble gas) needs fresh justification | International CCMHD literature heavily favors He, Ar, Xe, He-Xe. N₂ is much less studied. Aurora's N₂ choice was driven by safety/cost rather than σ optimization. Retired by: working fluid trade-off study with explicit σ × u² × B² × power-density × cost analysis. Several weeks of analytical work; should happen before any demonstrator hardware commitment. |
| Risk 5 — First-of-a-kind capex at integration scale | The documented reason the 1993 DOE EDP program was terminated. Architecture 4's components are now commercially-supported (HTS magnets, AM, pulsed power), but system integration at multi-MW scale is still FOAK. Retired by: bottom-up cost model from validated component vendors; compare against benchmark sCO₂ Brayton TEAs. |
| Risk 6 — Sakhalin-to-continuous operation gap | Sakhalin demonstrated 510 MW for ~7 seconds. Architecture 4 needs 8,000+ hours/year continuous. Hardware changes between modes not fully characterized in available literature. Retired by: detailed sub-system thermal-cycling and steady-state-vs-transient analysis comparing Sakhalin design with Architecture 4 design. |
| Risk 7 — Cs/refractory metal high-T compatibility | No >1,000-hour data in Cs-vapor at Architecture 4 operating temperatures. Photocathode UHV practice does not transfer. Retired by: combined with Risk 1 electrode lifetime program. |

# 4. Risk Retirement Development Plan

The risk inventory translates to a sequenced development program. Risks are retired in the order that makes subsequent investment decisions tractable.

### Stage 1 — Analytical Risk Retirement (months 0–6, ~$1–2M)

- Working fluid trade-off study (Risk 4)

- Bottom-up component cost model (Risk 5 partial)

- Sakhalin-to-continuous gap analysis (Risk 6)

- Architecture 4 σ × B² × u design space mapping using OpenFOAM

- HTS magnet engagement with CFS / Tokamak Energy

- AM electrode design with specialty metallurgists

### Stage 2 — Component-Level Validation (months 6–24, ~$5–15M)

- Electrode lifetime test program in Cs-vapor at temperature (Risks 1, 7)

- Cs handling sub-system bench-scale demonstration with photocathode community consultation

- Pre-ionization power supply integration testing with industrial plasma vendors

- Sub-scale (kW class) MHD channel testing for σ verification (Risk 3 entry point)

### Stage 3 — Sub-Scale Integrated Demonstrator (months 24–60, ~$30–80M)

- 1–5 MWe closed-cycle supersonic Faraday demonstrator (Risk 2, integrated)

- Validates: continuous operation, supersonic flow, non-equilibrium σ at scale, Cs handling at scale, magnet integration

- This is the Architecture 4 equivalent of the 1993 EDP, but at 40–100× smaller scale — what makes FOAK risk tractable

### Stage 4 — Commercial Pilot (months 60–120, ~$200–500M)

- 20–50 MWe pre-commercial deployment with strategic customer (data center, industrial CHP, naval auxiliary, etc.)

- Validates: lifetime at scale, integration into customer thermal/electrical ecosystem, economics at deployment scale

### Stage 5 — Commercial Deployment (year 10+)

- Full commercial product based on validated pilot performance

# 5. Strategic Positioning

### Market positioning

Mid-scale (5–50 MW) stationary continuous power, with specific value to customers who place premium on: (a) no rotating high-T turbomachinery, (b) high temperature capability, (c) modular scaling, (d) mobility / containerized deployment, (e) carbon-free dispatchable power when paired with green H₂ or coupled to closed combustion + CCS.

### Competitive set

- Below in capex: CCGT bare ($1,000/kWe). Architecture 4 wins on T capability and rotating-machinery-free operation in some segments.

- Comparable in capex: sCO₂ Brayton ($1,500–2,500/kWe), CCGT+CCS ($2,500–3,000/kWe). Architecture 4 differentiates on application fit.

- Above in capex: SMR nuclear ($5,000–8,000/kWe). Architecture 4 wins on capex but competes against nuclear's longer-lifetime/zero-fuel-cost story.

### Honest expected landing zone

$2,000–3,500/kWe at 50–55% efficiency, producing 5–50 MW per unit, in a competitive band rather than a dominant one. The strategic question is not ‘can Architecture 4 beat CCGT’ — it cannot, on pure capex. The strategic question is ‘is there a market segment that values Architecture 4's specific advantages enough to pay a moderate premium.’ This must be answered through customer engagement during Stage 1.

### Funding strategy implications

- Stage 1 ($1–2M, analytical): small grants, internal funding, or seed investment

- Stage 2 ($5–15M, component validation): Series A class commercial funding or DOE ARPA-E grant

- Stage 3 ($30–80M, sub-scale demonstrator): Series B class plus strategic partnership, or DOE BIL/IRA program

- Stage 4 ($200–500M, commercial pilot): anchor customer commitment plus debt financing or strategic acquisition

This funding sequence is materially different from the 1993 DOE EDP path (federal program from concept through commercial demonstration). Architecture 4's case rests on a commercial-first development sequence with selective public funding for component validation, rather than a federal-first sequence locked to DOE budget priorities.

# 6. What Architecture 4 Is Not

- Not a substitute for CCGT in cost-driven commodity power markets

- Not a near-term commercial product (10+ years from current state)

- Not de-risked — the four-phase exercise reduces risk perception but does not retire any of the seven identified risks

- Not a single-architecture bet for the Aurora technology set; benefits from cross-cutting work shared with other architectures

- Not the highest-priority architecture in the Aurora set automatically — comparable exercises for Architectures 1, 2, 3 may surface different prioritization

# 7. Architecture 4 Prior Art Snapshot

Single-page reference summary of where each design dimension stands after the four-phase prior art exercise.

| Design Dimension | Status | Source / Action |
| --- | --- | --- |
| Closed-cycle MHD physics | Demonstrated (sub-MW) | NASA Lewis |
| Multi-hour MHD operation | Demonstrated (open-cycle) | U-25, CDIF |
| Non-equilibrium σ enhancement | Demonstrated (sub-MW) | Tokyo Tech |
| Closed-cycle without slag | Architecturally favored | By design |
| Modern HTS magnets at 15 T | Commercially validated | CFS, Tokamak Energy |
| Modern CFD design tools | Validated | Marzouk OpenFOAM solver |
| Cs handling engineering | Mature in adjacent industry | Photocathode community |
| Pre-ionization power supply | Commercial COTS | SiC/GaN pulsers |
| Refractory metal AM | Production-ready | Multiple vendors |
| Supersonic Faraday at scale | Demonstrated (pulsed) | Sakhalin 510 MW |
| Supersonic closed-cycle integration | Never demonstrated | Risk 2 — demonstrator |
| Multi-MW non-equilibrium σ | Never demonstrated | Risk 3 — sub-scale test |
| Cs-vapor electrode lifetime | No data >1,000 hours | Risk 1 — test program |
| Working fluid choice (N₂ vs noble) | Needs fresh analysis | Risk 4 — Stage 1 study |
| FOAK capex at integration | Bounded but real | Risk 5 — cost model |
| Continuous from pulsed | Engineering gap | Risk 6 — gap analysis |

Net Assessment

Architecture 4 is a defensible engineering bet with identifiable, bounded risks. The 1993 DOE program's broad ‘unresolved technical issues’ do not apply — Architecture 4's risks are specific and addressable through a structured development program. The strategic case depends on customer-facing market validation in Stage 1 and on cost discipline in Stage 2-3 to keep FOAK capex within the $2,000–3,500/kWe landing zone.

This synthesis derives from a four-phase prior art landscape covering: 9 direct heritage entries, 6 supporting work entries, 7 disrupting work entries, 6 adjacent inspirational entries, plus 24 cross-cutting register items spanning the broader Aurora technology set. The full prior art document accompanies this synthesis as a working reference.